Translated by Denis Godfrey, G0KIU (mni tnx)

There may be a more recent translation, but I haven't got one.

73s Rene, DG0FT

------------------------------------------------------------------

TFPCX v1.10  Resident AX.25-Controller for PC without TNC

(Free for Radio Amateurs, No Commercial Usage)

Rene Stange, DG0FT (ex Y51GE)

INTRODUCTION

TFPCX  ((T)he  (F)irmware (PC) E(x)tended)  is compatible

with TheFirmware from NORD><LINK and runs resident in the

Background  as  an AX.25-L2-Controller on IBM  Compatible

PC's (Not on ATARI ST).  The output is able to be sent to

a simple Modem (e.g. BayCom-Modem von  DL8MBT und DG3RBU)

and  via  a Software Interrupt able to  communicate  with

known  Terminal Programs such as:  SP from DL1MEN and THP

from DL1BHO in the WA8DED-Hostmode, which on this account

are   able  to  be  employed  without  a  TNC.   In  this

description  the  Operation with SP  will  be  explained.

TFPCX is developed from the KISS-Driver which belongs  to

the  distribution of SP and will be  employed  similarly.

The  KISS-Interface  of the TFPCR has been replaced  with

HDLC-Send  and Receive routines,  so that with the  TFPCX

(still!)  will be able to operate with no KISS-TNC's  and

Controller (e.g.  PK232).   As Modem-Ports all COM-   and

LPT-Ports (e.g.  for Digicom-Modems) are usable. In short

is  able to be controlled with only a Modem.  The Program

is  also  operable  in Terminal-Mode (Stand  alone)   and

contains for that purpose a simple Terminal-Routine.

Preface

-------

After the most positive reaction to the release of TFPCX v1.01 and

the further development of DAMA-Digipeaters I saw myself obliged

to bring TFPCX to the 'latest' level and to make compatible to TF

2.3b. Nothing has altered from the basic Function. As a further

innovation TFPCX now has a Soft-DCD (Programmable Squelch Barrier)

and makes possible an internal self connection for internal Test

work. Also for non-DAMA operation this version offers advantages.

I must however advise, that I am not able to guarantee, that the

TFPCX will function problem free on all PC's. Especially on slow

Computers it gives partial Receive- and Send Problems. XT's with

operating speeds under 8 MHz are not usable or only within

limits.  This is not a commercial product, so I accept no

responsibilty for its non-operation. In the Majority of cases the

TFPCX-User there will be no appearance of problems.  Conclusive

evidence of its Functioning is only able to given by a Test on the

individual PC. Nevertheless TFPCX places negligible demands to the

compatibility of the employed Port compared to Baycom, so that it

also runs to some extent on Computers which are not BayCom-

suitable.

With confusion about the operation of TFPCX in connection with SP

on [1] reference (read under TFPCR). I start out in this

description with the assumption that there is already a functional

Modem on hand. Who employs a Baycom Modem, is contained in [2]

Advice for the setting to work.

I thank all YL's and Om's, who have contibuted and contribute with

the advice for the further development of the TFPCX, also if

number of their ideas are not yet realised and a Time delay

for the reply is important. On this account I offer all, with

appearing problems to have a look in this documentation first,

then for many questions here is given a certain answer. A special

thanks intended for all contributions!  Also for myself, I have

rejoiced at the positive reactions and support of the Radio

Amateurs, that has flowed in for development of TFPCX,

respectively who helped to get the TFPCX to this level (Sigi

(DL1MEN, NORD><LINK, the BayCom-Team and others.).

73s from Rene, DG0FT @DB0KG      Strausberg, 26th. December 1991

Innovations since v1.01

-----------------------

- compatible with TheFirmware 2.3b DAMA from NORD><LINK

- Soft-DCD (programmable Squelch Barrier) with switchable Send-/

Receive notice (Option '-NC', Command '@C' for the equalisation)

- internal Self Connects possible, all transmitted Frames will

also be inspected as received Frames (Self Connect will be

urgently recommended)

- automatic SSID-selection with Multi-Connects of a Station, if

installed SSID already created

- 600 instead of 400 free Buffer, TFPCX now occupies 54K in the

RAM

- Option '-ND' for the delaying of Disk Access during the Send and

Receiving (Important with Problems)

- Bug remedied, which on 486's made the Unloading impossible

- Option '-B9600' no longer exists (also for 486'ser too much)

Copyright

---------

TFPCX may be passed freely to others for the usage in Amateur

Radio. It is not permitted for the Program to be used commercially

or to be sold commercially.

A guarantee for a correct functioning will not be given. The User

has no right of recourse, in the event of a Malfunction of TFPCX.

The Program TFPCX has been developed by DG0FT by utilisation of

the Source Code of the PD-Programs TFPCR v1.60 from DL1MEN and the

PD-Sources of the TheFirmware 2.3b from NORD><LINK (Original

version from DC4OX, DAMA from DL8ZAW, modifications from DB2OS,

DF2AU, DF7ZE,  DK6PX,  DL1BHO, DL1MEN, DL4YBG, DL9HCJ and others)

Quick Start

-----------

For all who prefer to find out for themselves, a BayCom Modem and a

normal Ultra Short Wave-Rig with 1200 baud is required.

- he who employs an XT, should read further because tests should be

made to ensure that the Computer is fast enough.

- SP (from 5.02) installed, then whether one uses TFPCR

- TFPCR.COM replaced by TFPCX.EXE

- BayCom Modem installed on a COM Port

- 'TFPCX PCOMn' called (n is the Number of the Port)

- Start SP

- If there are problems then one should read further

- With 'TFPCX -U' :- TFPCX is able to be unloaded.

Principle of Operation

----------------------

TFPCX is a TSR Program, that must be called before activating SP and is

installed resident in the RAM. If the foregoing is satisfactory, the

DOS-Command prompt returns and the next Program will be able to be

started. The System behaves as if a TNC was connected, external

connects are possible and it stores all incoming Information. As soon

as the Terminal Program is started, the received Text will be displayed

on the Monitor. The communication with SP follows by way of a Software

Interrupt. Here no difference in principle to the employment of TFPCR

should be encountered at this point. TFPCX is similar to the Program L2

of the BayCom System by Flori (DL8MBT) and Johannes DG3RBU.  Here SP

takes over the role of the employed Program SCC.

What is truly new in comparison is, that the TFPCX has the ability to

control the Modem and is in a position to receive and send AX.25-Frames

without a TNC. A different procedure will be employed here, to that in

BayCom, which also has advantages as well as disadvantages. The

requisite normal timer produces the System Timer (8253 Channel 0),

activates the 3,600 times per second Interrupt 8 (For 1200 Baud), which

already places a known claim on the Computing performance of the PC.

Therefore the TFPCX on normal IBM XT's with 4.77 Mhz operation is not

compatible with SP, 8 Mhz should however be sufficient. The greatest

problem is that the Interrupts of the timer may not be protracted too

long, so that an orderly reception of Packet is possible, and the

transmitted Frames will not be mangled. That is able to happen, e.g

with Disk-Access.  A crude summary of what is able to be awaited is

given in the following Table (Without Guarantee).

PC        XT        XT        286       386

MHz       5         8         12        20

Baud

300      *         *         *         *

1200      ?         *         *         *

2400      /         ?         *         *

4800      /         /         ?         *

*    Operation possible

?    Operation possibly with some restrictions

/    Operation not possible

Another problem in this connection are Programs, which themselves

make demands on the timer. (e.g. MS-Windows, most Mouse Drivers or MS-

Quick 2.51 that I have used.) These Programs are not able to be used as

long as TFPCX is active. Contravention will be punished with false

System Clock running, extreme retardation of the Computer or System

Crash.

Up to now it has been comments about disadvantages, nopw what is more

positive: what is overlooked is that SP through it's longer development

history offers more features than BayCom, the TFPCX does not require a

fully compatible COM-Port because the Serial Controller will be

employed as a Simple In and Out-Latch. The Port must not be Interrupted

and it is able also to employ all LPT Ports as an Interface. (e.g. For

Digicom Modems, deliver the 5V level) Also new hope for people, who

have had no luck with BayCom !

Modem Connections

-----------------

It was not possible for me, to produce a new Modem yet, that is able

to be substantially better. So I have retained the Quasi-Standard

BayCom Modem, that many YLs and OMs already own.   Additionally the

possibility remains to connect another Modem (e.g. from Digicom) via a

Centronics Port (from me the original variation) Here are the

Connections of the Port.

COM Port [2]

Signal    25 pin    9 pin   Operation

DTR         20        4     Data Send +/- 10V

RTS          4        7     PTT, High active, -12V=RX  +12V=TX

CTS          5        8     Data Receive

GND          7        5     Ground.

TXD          2        3     +12V for Baycom-Modem

LPT-Port

DATA         7              Data Send, TTL Level

DATA         9              PTT, High active, 0V=RX, 5V=TX

BUSY        11              Data Receive

GND         18-25           Earth.

The Firmware v2.3b

------------------

Here shortly, something will be said about the realisation of TFPCX

v2.3b. The Firmware is able to be employed in two types of

operation. (Terminal and Hostmode.) which is also possible by every

TNC. The difference to a normal TNC is only that, the Data exchange

ensues not by the serial Port but by Software interrupts with different

subordinate functions. In the appendix this Software Interface is

described.

are listed.

The Firmware offers a series of Commands, which with the ESC-Key will

be initiated and executed with <RETURN>. In the Appendix a short

collection of the realisable Commands are to be found. In [3] the

Firmware is documented. The Commands 'A', 'E', 'H', 'QRES', 'Z', '@F'

and '@M' do not exist.

Here follows an explanation for circumspection of individual Commands:

Command    Remarks

C       It is no longer important to manually install the different

SSID's for the Multi-Connects of a Station (e.g. Digi-

access). TFPCX raises this automatically to a maximum of 15

if the one already installed will be employed This Function

SP remains concealed of course.  Possibly in the Status Line

and in other places stands the False SSID. When at the

start, the SSID must be self installed. TFPCX is able to

handle 10 Connections simultaneously. Internal Connects are

possible.

F       The Frack-Parameter to be given alternatively in Seconds-

or  10 ms-Increments. Values smaller than 16 will not be

ignored, as with the 'normal' TF 2.3b but in the new

Uniformity converted. Thereby the old Config-Files must be

unconditionally altered.

P       Here with DAMA-Operation the non-DAMA value will be

indicated but P=255 used.

U       The Unattended Mode is able to be switched on, when no

CTEXT is defined (Standard Installation).

The Report 'NO MESSAGE AVAILABLE' no longer given

@C      With this Command the Soft-DCD will be balanced.

(See further extract below)

@T4     This Command is on hand only out of Compatibility Grounds,

give the T2-Start Value for DAMA-Operation and the correct

time, which will be waiting to be verified to a received

Frame. It is from the original Firmware and in the meanwhile

has been removed again.

@U      With this will be determined, whether Unproto-Frames will be

transmitted with fixed Poll-Bit (Standard) or without.

A few words about DAMA: As soon as on a Port a Connection to a DAMA-

Master (Digi-Entrance) is encountered, TFPCX will be automatically

switched in the DAMA-Slave-Mode and then only sends when it receives a

Frame from the Master, then of course all outstanding Frames on all

Ports. It is not required for DAMA special Parameters to be installed.

thereby alternative operation is possible problem free. With the 'B'

Command is is able to establish, whether DAMA is switched on (Value in

Brackets larger than 0). The Frames received from the DAMA-Master (and

only these, not the others) contain in the Monitor the addition

'[DAMA]'

With Tests is has been shown, that the DAMA-implementation in the

Firmware 2.3b is not yet optimal. It is found for example, that one

with Multi-Connect by reason of too short a Frack, will be noticed by

TheNetNode-Digi before all others, what was not intended to be so by

the DAMA-Designers is. But it is in every case better to operate a non-

optimal DAMA-Slave than not at all.

Call of TFPCX

-------------

TFPCX will be activated by the following Command Lines.

TFPCX [ -N  ]  [ Load options | -T | -U ]

All Parameters are optional, will be preceded with '-' and will be

separated one from another by a space. Within an option no spaces are

permitted. Upper/Lower Case is not significant. In the first place all

options shortly will be listed out in the form, as they are also

obtainable in the Help Text with "TFPCX -H". Load options are only

relevant with the resident load of the TFPCX and valid until the

unload.

-N    No Messages

-T    Terminal Mode

-U    Unload

Load options:                        Default

-Pport[:xxx]  Modem port (address)   COM1

-Bnnnn        Baud rate              1200

-Ixx          TFPCX interrupt        FD

-F[file]      Send file to TFPCX     TFPCX.INI

-D            Debug mode             Off

-NB           No blinking rectangle  Off

-NC           No carrier display     Off

-ND           No disk access if DCD  Off

[]    Issue is optional

|     alternative Issue

n     Deczimal Number

x     Hexadecimal Number

After the Start the following Report should appear

��������������������������������������Ŀ

�  TFPCX v1.10 (Dec 08 1991) by DG0FT  �

�     TF v2.3b DAMA by NORD><LINK      �

�    Free for non-commercial usage     �

��������������������������������������Ĵ

� COM1, 1200 Baud, Int FD, 10 Channels �

����������������������������������������

and the DOS-Prompt visible. TFPCX is now installed and occupies circa

54 KBytes of the Main RAM. Here is the meaning of the options.

-PCOMn  (BayCom-) Modem on COMn (Possibilities are COM1 to COM4)

-PLPTn  Modem on LPTn (Possibilities are LPT1 to LPT4)

The base address of the Port will be taken from the BIOS Data range and

must be obtained from there. Most BIOS Versions neglect that for COM3

and COM4. In this case the address will be explicitly set in the

Form '-Pport[:xxx]' auch explizit gesetzt werden.

Example

TFPCX -PCOM3:338

With this call a BayCom-Modem will be employed on COM3, whereby the

Base Address will be 338H. This Address must be obtained from The

Serial Port description. The number of the Port (here cosequentlt the

3) will be ignored if an Address will be given, but must never the less

lie between 1 and 4. As Base address is tolerated the range 100H to

3F8H. The IRQ of the Port is of no interest to TFPCX and will not be

used.

-Bnnnn    Baudrate on the Modem (300, 1200, 2400, 4800)

Which Baud Rate is possible on a PC, depends on its operating

capability (see Table above). If it will not be worked with 1200, it

is also not the Normal BayCom-Modem.  Operation with 300 Baud the

System Clock loses a half minute in an hour.

-Ixx      Software-Interrupt for Communication with SP (40-FF)

Interrupt FD will be employed as standard  prcatice. A difference is

only important, if this Vector will be used by other Programs.

-N        Suppress Messages of the Program

If the Messages of TFPCX at the Intialisation are not aesthetically

sufficiently modest, it is able to suppress them. But Error Reports

will still appear.

-NB       Switch off Status Flash

When TFPCX has buffered unread Information or Status Reports and is not

in HOST Mode (Also if SP is not running), a square flashes in the top

right hand corner of the Screen, that for example draws attention with

a new Connect. SP may remove this by means of ESC 'DOS' or ESC '!,

because TFPCX in this case remains in Host Mode. It is now able to

start SP and react to a Connect. The Flashing is able to be suppressed

with this Option, in the case where it distracts.

-NC       Supress display of the Channel Status

Whilst TFPCX is in the Host Mode (also SP runs) an 'R' will ne

displayed in the top right hand corner of the Screen, immediately when

a PR-Signal is received and an 'S' displayed, when the Station

Transmits. The appearance serves for equalisation of the Soft-DCD. With

this Option the appearance is able to be switched off.

-ND       Suppress Disk-Access verzgern, if immediate receive or send

will be actioned (Emergency Expedient)

In the case where there are Receive problems with Disk-Access (Packets

will not be faultlessly decoded) it is able with this option to

prevent, that a Disk Access will be carried out, during an adjacent

Signal. That leads of course to a somewhat 'unusual' conditions,

because the Computer then shows 'hang up' until the QRG is again free.

This option should only be used in an emergency. If TFPCX is not in

Host Mode all Disk Access will be immediately executed.

-D        Test Mode (Debug)

Causes with each Timer-Interrupt an edge change at the entry of the

Loudspeaker. With 1200 Baud a 1800 Hz-Tone will be heard

(Baudrate*1.5). The tone should be 'halfway' clean. Crackling

originates, if the Interrupt will be delayed. If a single crackling

occurs the Computer is overloaded. Here the dividing line is certainly

very difficult to draw, a 'certain Ground Noise' must not yet damage

the Function.

-Ffile    File for the Parameter Installation (will not normally be

used, if the File Name fails, TFPCX.INI is effective)

If required, this File will be read at the Initialisation and in the

Terminal-Mode sent to the Firmware, in order to pre-Install the

Parameters. That may be of special interest, if the Program is

required to be allowed to run in the background without starting SP or

for the Operation in the Terminal-Mode. Normally this option is able to

be dropped, because SP itself normally undertakes Intialisation. The

File will be sought in the immediate Directory, when no full Path name

will be issued. The Character '^' will be transformed into an Escape,

with which in Terminal-Mode Commands will be commenced. Line feeds will

be filtered out, so that 'file' is able to be created with a normal

Editor. An example would be:

TFPCX.INI:

^F 300

^I MYCALL

^N 16

^O 2

^P 64

^R 1

^T 25

^U 1 Willkommen in ???? - TFPCX v1.10 - SP offline

^W 25

If TFPCX entirely without Parameters, so it will be assumed that

'TFPCX -PCOM1 -B1200 -IFD' has been used.

Installation of TFPCX

---------------------

(1) SP so installed, that it is able to work with TFPCR

(see [1]).  Especially, the following lines must be contained in the SP

CFG=TNCS:1

CFG=PORT0:5

CFG=CHANS0:10

SP-Versions before 5.02 should not be used, because the TFPCR-Interface

does not yet exist in the correct form. Where TFPCX is required to be

operated in High RAM,  'CFG=PORT0:5H must be used.

This variant requires that no further TNC will be employed by TFPCX.

However Multi-TNC Usage by the employment of TFPCX functions.

Therefore TFPCX is able to be employed only for one HF-Port and TFPCR

is not simultaneously loadable. One must be clear that the Interrupts

of the Serial Port are impossible to be made on which TNCs are

attached, so that impeccable AX.25-Frames will be received and

transmitted. On this account a faster Computer is already required.

All that belongs to TFPCR (TFPCR.COM, UNTFPC.EXE, AUTOKISS.CFG) is able

to be erased (the original Diskette naturally in safe keeping).

Therefore the Data TFPCX.EXE should be copied into the same Directory

that SP.EXE resides.

(2) Now for the Modem on COM1-4 or LPT1-4 to be connected and TFPCX

called. If COM1 is not used, the Port Parameter must be given (see

above). If the System hangs for an extremely long time after the load

(before the Disk is accessed): is the PC overloaded with the Baud Rate?

(3) Next SP will be started. No Report Case.

TNC#0 - (TFPCR / FE) in Hostmode

displays, has something gone wrong with the SP-Installation or TFPCX

has not loaded. Newer SP-Versions are able to distinguish TFPCR and

TFPCX  and eventually show a somewhat varying Report. Now one looks if

there is a QRG on the PR-Operation, and switches on the Monitor with

<ESC> 'MISUC'. If in spite of incoming signals no Frames are able to be

indicated, quit SP again, remove TFPCX with 'TFPCX -U' from RAM and re-

start it again with the Option '-D' added.  Now a 1,800 Hz tone should

be heard from the Loudspeaker of the PC. (with 1,200 Baud). If all is

quiet, the PC has either an incompatible Timer (Then anything further

is pointless) or the 'Chirper' simply does n't work. (I have that once

already). Now SP will be called again, and accurately pick up one's

ears. If a single Crackle Concert sounds, the Computer is not fast

enough to react to the Interrupts of the timer (during the loading a

Crackle is not detrimental). If the sound is relatively clean, could

the error lie with the Modem or the HF-Technique.

(4) If the Monitor operates, one is able to attempt a Connect. Thereby

the PTT of the Transmitter should be observed. All should now function

correctly. It is also conceivable, that reception fails, but mutilated

Packets will be sent.

Where SP is usually with TNC2 perhaps it will be some other Time

constraint. So flash the QRG-Indicator on the Monitor Screen and the

alter the Connect Bell.  This constraint is is completely normal,

would be able to be remedied in SP and is able to be taken will in

hand.

TFPCX removal from RAM

----------------------

With the Command 'TFPCX -U' TFPCX removes itself from the RAM. It is

not necessary to load an extra Program. The Communications-Interrupt

will be automatically found and will not require to be declared.

Terminal-Mode

-------------

With 'TFPCX -T' a simple Terminal-Program will be started, with which

one  is able to operate without SP. Before this the resident TFPCX is

loaded (as described above). One must call the Program twice with

different Parameters in order to arrive in the Terminal-Mode. With

ALT-X, exit from the Terminal-Program will be achieved. Previously one

should switch the Monitor off with <ESC> 'MN', if was active, because

otherwise unimportant saving of the Buffer will occur , and problems

result with the eventual starting of SP. If it is required to use only

the Terminal-Mode (e.g. on slow Computers) it is an advantage to employ

the option '-F' for the installation of Standard Parameters.  The

following Batch File helps for this:

TERM.BAT:

@ECHO OFF

CLS

TFPCX -N -F

TFPCX -T

TFPCX -N -U

Soft-DCD

--------

TFPCX  now has a programmable Noise Barrier (Squelch). It is able to

to allow the Squelch of the Receiver to be be fully open and TFPCX

decided itself, whether a genuine PR-Signal is being receiver or only

noise.  The Soft-DCD will be controlled through a new Command '@C',

with which one install the chosen threshold. As a Parameter a number

from 0 to 63 will be issued. With '@C0' the Soft-DCD is switched off

and TFPCX retains as before. This is the Standard Case. With all other

Values the Noise Barrier on. At all times the larger the value is, the

stronger is the Soft-DCD recorded. For the facility of the adjustment

an 'R' will be indicated in the top right hand corner of the Screen

when a signal is recognised, and an 'S' when the Station sends itself

(only when SP is running). With too small a value the DCD Indicator

flickers, with too large Values Signals will be no longer correct and

too slowly recognised. For the best raise the Parameter so far and

listen to QRG, up to the true notification. With that an eventual

compromise will be found. A good Value is '@C25'.

IMPORTANT

The Soft-DCD should only be employed if one does not have a Hardware-

Squelch that is in good order, because the recognition is eventually

not problem free. The conditions with other Baud Rates than 1200 have

not been tested. For the best probe and decide yourself !

Error Correction

----------------

In case there are problems with bad reception or ones own outputs are

not able to be decoded by other Stations lies the eventuallity thereon,

that the Timer-Interrupt Routine for TFPCX's 'life importance' will not

always be called up correctly timed, because the Interrupt was barred

to long. This problem stands before everything with Disk Access, Access

to the Extended Memory (e.g. RAM-Disk), Toggled in the Protected Mode

(e.g. EMM386) and slow Keyboard Drivers. In the last case, the Driver

CKEYGR.COM from the SP-Diskette helps. Also many TSR Programs bring

problems. One is only able to attempt, to dispense with all of these

Problem sources, but what is not always possible. In case of emergency

the option '-ND' also helps, if Disk Accesses are to blame.

With the BayCom-Modem there are possibile Problems through the

stabilised Voltage supply with the BayCom Modem. in connection with

TFPCX. Here the TXD-Control remains static on approximately +12V whilst

Baycom delivers a time signal on this condutor. For this reason the

supply voltage of the Modem lies somewhat higher and the potentiometer

Pin 7 of the TCM3105 delivers a Voltage differing from the ideal value.

In this case a new balance of the potentiometer is necessary. (see [2]).

*   *   *

Anyone who has an interest for the Program, send an empty Diskette and

a self address and an adequately stamped envelope to:

Rene Stange

O.-Grotewohl-Ring 34

15344 Strausberg

Germany

Possible Diskette-Formats: 3 1/2" 720K oder 1,44M

5 1/4" 360K oder 1,2M

Please particularly observe the foregoing Text!  I make no financial

gain, it is not possible for me to pay the postage out of my own pocket

or more expense to supply a Diskette for copying, to place in the

completed envelope and take it to the post.

APPENDIX

Functions of the TFPCX-Interrupt

--------------------------------

There exists four lower functions, which by the Value in the Register

AH is able to be selected by a call. Posible Parameters will be

delivered into AL. AX contains by return, the result or FFFF if a non-

existent Function has been selected. All presented Characters should be

read, before the next input should be made.

AH = 1    Request, whether a request Character present

Returns   AX = 0   No Character present

AX = 1   Character present

AH = 2    Character request, only calls if Function 1 has

imparted, that a Character is present.

Returns   AL        Character code

AH = 3    Output of a Character to the Firmware

Parameter: AL        Character to be output

AH = FE    Request for the TFPCX-Version Number

Returns   AH = 1    Main Version Number

AL = 0AH  Sub-Version Number

Three Bytes after the jump into the TFPCX-Interrupt-Routine stands the

Identification 'N5NX', which operation Interrupt on hand the Terminal

Program is able to self ascertain.

Commands of TFPCX

-----------------

COMMAND          PARAMETER                 DESCRIPTION

B(120)         1...600                DAMA-Timeout-Time (Seconds)

0                   DAMA-Switch-on Blocked

C     C/Sign1 [C/Sign2...C/Sign9]     Connect-Path

(in Channel 0: unproto)

D                                     Sever connection

F (300)         16...65535            Start Value for SRTT (10ms)

1..15                Start Value for T1 (Seconds)

G                [0]                  Get Info in the Host-Mode

[1]                  Get Status in the Host-Mode

I             Call Sign               Own Call Sign

JHOST (0)         0                   Terminal-Mode Switch on

1                   Host-Mode Switch on

L              (0...10)               Status report for the Channel

M (N)           NIUSC+-               Monitor Operation

N (10)          0...127               No. of requests (0=infinite)

O (2)            1...7                No of unacked Packets

P (64)          0...255               p-Persistance Value

R (0)              0                  Digipeat Off

1                  Digipeat ON

S (0)            0...10               Channel Number (0 = Unproto)

T (30)          0...127               Wait Time (DW) for PTT 10 ms.

U (0)           [Text]                Connect Text suppressed

[Text]                Connect Text ON

V (2)              1                  Protocol Version 1

2                  Protocol Version 2

W (10)           0...127              Timeslot for p-persistance

10 ms.

X (1)              0                  PTT for TX Suppressed

1                  PTT for TX Free

Y (10)           0...10               Maximum quantity of Connections

@A1 (7)          0...65535            SRTT-Smoothing, if RTT rises

(SRTT'=(A1*SRTT+RTT)/(A1+1))

@A2 (15)         0...65535            SRTT-Smoithing, if RTT falls

(SRTT'=(A2*SRTT+RTT)/(A2+1))

@A3 (2)          2...16               Factor for T1 (T1=A3*SRTT)

@B                 0                  Shows quantity of Free Buffers

@C                 0                  Software-DCD OFF

1...63               Threshold for Software-DCD

@D (0)             0                  Full Duplex OFF

1                  Full Duplex ON

@I (60)            0                  IPOLL OFF

1...256              max. length of IPOLL Frame

@S                                    Immediate Link Status

@T2 (150)        0...65535            Timer T2  (10ms)

@T3 (18000)      0...65535            Timer T3  (10ms)

@T4 (10)         0...65535            Timer T2 with DAMA (10ms)

@U  (1)            0                  Unproto-Frames without Poll

1                  Unproto-Frames with Poll

@V                 0                  Call Sign Check OFF

1                  Call Sign Check ON

[] Optional Parameters

() Standard installation

Bibliography

------------

[1] DL1MEN: Manual of Eskay Packet (SP)  from v5.02

[2] NORD><LINK: Documentation for the Firmware 2.3b

[3] DL8MBT, DG3RBU: BayCom - Documentation for the Modem